Class: RBE 577 – Machine Learning

Assignment: HW1

Students: Camilo Girgado & Jason Patel

9/28/2025

Encoder-Decoder Neural Network with Custom Loss Functions

Methodology	1

Network Architecture	1

Data Generation Process	2

Multi-Component Loss Function	2

Training Strategy	3

Hyperparameters	3

Results and Key Findings	4

Conclusion	4

This report analyzes the autoencoder neural network designed for maritime thruster control system as described in our reference paper, Constrained Control Allocation for Dynamic Ship Control. As part of the assignment, we worked to replicate both the methodologies and the results that were used and seen in the paper. The system itself uses a 3-layer encoder-decoder architecture to learn compressed representations of 3-dimensional force inputs while incorporating multiple constraint-based loss functions to ensure physically realistic and operationally safe thruster commands.

## Methodology

### Network Architecture

We implemented solution a symmetric autoencoder architecture with the following structure:

Encoder Path: 3 → 16 → 8 → 5

- Input layer: 3 dimensions (representing force components F1, F2, F3)

- First hidden layer: 16 neurons with ReLU activation and 10% dropout

- Second hidden layer: 8 neurons with ReLU activation and 10% dropout

- Bottleneck layer: 5 neurons with ReLU activation (encoded representation)

Decoder Path: 5 → 8 → 16 → 3

- Mirror architecture of encoder

- Reconstructs original 3-dimensional input from 5-dimensional encoded space

- Uses ReLU activations and 10% dropout in hidden layers

### Data Generation Process

As described in section 3 of the reference paper, we synthetically generated the training data using the following physics-based approach:

- Input Parameters: Random sampling from normal distributions for:

Figure 1 – Physical Parameters

  - Force components: F1 ∈ (-10000, 10000), F2 ∈ (-5000, 5000), F3 ∈ (-5000, 5000)

  - Angles: α2, α3 ∈ (-180, 180)

- Transformation Matrix: A 3×3 transformation matrix τ is constructed using trigonometric functions of the angles and geometric constants (l1=-14, l2=14.5, l3=-2.7, l4=2.7)

Figure 2 – Transformation Matrix

- Output Calculation: The result is computed as τ × F, representing the physical relationship between input forces and system response

- Dataset Size: 1,000,000 samples for comprehensive training coverage

### Multi-Component Loss Function

The system implements a sophisticated loss function combining five components:

L1 - Reconstruction Loss: Standard MSE between input and decoded output

L2 - Thruster Command Magnitude Limits: Penalizes commands exceeding physical limits

L3 - Rate Change Loss: Constrains rapid changes between consecutive commands using L3 norm

L4 - Power Consumption Loss: Minimizes power usage via |u|^(3/2) penalty on force components

L5 - Azimuth Sector Loss: Prevents thruster operation in forbidden angular sectors (-100° to -80° and 80° to 100°)

### Training Strategy

- Optimizer: Adam with learning rate 0.001

- Batch Size: 100,000 samples

- Early Stopping: Terminates if test loss exceeds training loss for 200+ consecutive epochs

- Model Persistence: Saves best performing model based on training loss

- Train/Test Split: 80/20 random split with fixed seed for reproducibility

### Hyperparameters

Network Architecture Parameters

- Input Dimensions: 3

- Encoder Hidden Layers: [16, 8]

- Bottleneck Dimensions: 5

- Decoder Hidden Layers: [8, 16]

- Output Dimensions: 3

- Activation Function: ReLU (all layers)

- Dropout Rate: 0.1 (10%)

- Data Type: torch.float32

Training Parameters

- Learning Rate: 0.001

- Optimizer: Adam

- Batch Size: 100,000

- Maximum Epochs: 1,000

- Early Stopping Patience: 200 epochs

- Train/Test Split: 80/20

- Random Seed: 42

Physical Constraint Parameters

- Force Limits: F1 ±10,000, F2/F3 ±5,000

- Angular Limits: ±180°

- Rate Change Thresholds: [1000, 1000, 1000, 10°, 10°]

- Forbidden Sectors: [-100°, -80°] and [80°, 100°]

- Power Law Exponent: 1.5

## Results and Key Findings

- Large batch sizes and comprehensive early stopping mechanisms promote stable convergence and prevent overfitting.

- Dropout in both encoder and decoder helps reduce overfitting despite the large dataset, ensuring robustness.

- Each additional loss term enforces realism but may slow convergence or conflict with pure reconstruction accuracy. Further tuning of loss weights could improve performance.

- We noticed a difference in output when changing the hidden layer dimensions from 16–8 in the encoder and 8–16 in the decoder to 32-16 because this would directly affect the model’s capacity to learn. The larger hidden layers allow the network to capture more nuanced nonlinear mappings between inputs and outputs, which in turn would reduce reconstruction error.

Figure Loss for 3-16-8-5

Figure Training/Test Loss by Epoch for 3-16-8-5

Figure Loss for 3-64-16-5

Figure Training/Test Loss by Epoch for 3-64-16-5

## Conclusion

Both the reference paper and our script demonstrate how neural networks can be guided by physics-inspired loss functions to produce solutions that are not only numerically accurate but also operationally feasible. The encoder-decoder framework successfully learns a compact latent space for thruster commands, while custom losses enforce safety, efficiency, and physical constraints.